Chapter 4.  IR Remote Control Operation

4.1	Introduction

The VPON DVR can be used and configured with either the supplied remote control unit or with a keyboard and mouse. This chapter covers remote control operation. The VPON DVR must be in IR remote control running mode for remote control operation to function.

4.2	Main menu

Press the Menu button on the remote control to display the main menu on screen.

Use the Up and Down arrow buttons to select one of the four options and press the OK button to confirm your choice.

4.3	System setup

The System Setup menu enables you to change many different parameters of the VPON DVR. This section describes each option in the System Setup menu.

Use the Up and Down arrow buttons to highlight an option, and use the OK button to confirm your choice. You can move between pages using the Prev Page and Next Page buttons on the remote control.

4.3.1	Password protect

Use the Password Protect option to enable password protection, set administrator and user passwords, set the auto lock period and lock the system immediately. Enabling password protection prevents unwanted users from accessing or configuring the VPON DVR. The Password Protect menu has the following choices:

Enable - enables password protection.

Admin Pass - sets the admin password.

User Pass - sets the user password.

Auto lock After - enables password protection after the specified time interval.

Lock Now! - enables password protection immediately.

To set the user or admin password:

1.	Use the Up and Down arrow buttons on the remote control to highlight the Admin Pass option to set the administrator password or the User Pass option to set the user password.

2.	Press the Right arrow button to display the password entry screen.

3.	Press the keypad button on the remote control to display the on-screen keyboard.

4.	Use the Arrow buttons on the remote control to choose a letter and use the OK button to confirm you choice.

5.	When the new password has been entered correctly, press the keypad button on the remote control to turn off the on-screen keyboard.

6.	Press the OK button to confirm the new password.

When user or admin passwords are set and enabled, you will be prompted for a password when they attempt certain functions. Use the on-screen keyboard to set user or admin passwords as described.

To set the password protects function:

1.	Select the Enable option from the Password Protect menu.

2.	Use the Left and Right arrow buttons to toggle between Yes and No.

3.	Press the OK button to confirm your choice.

The autolock function in the Password Protect menu will lock the system after the specified period of inactivity.

To set the period of inactivity before the system is locked:

1.	Highlight the autolock option in the Password Protect menu.

2.	Use the Left and Right arrow buttons to increase or decrease the period of inactivity.

3.	Press the OK button to confirm your choice.

To lock the system immediately:

1.	Highlight the Lock Now! Function in the Password Protect menu.

2.	Use the Left or Right arrow buttons to confirm the operation.

4.3.2	Video

Use the Video menu to set all the video parameters for the cameras connected to the VPON DVR. There is one video screen for each camera with the following parameters:

Standard - sets the video signal format. Choose NTSC in the US, or PAL in Europe and most Asian countries. If you are not sure about this option, select Auto. VPON DVR will detect the video standard automatically.

Brightness- adjusts the amount of light or brightness for the selected camera according to your preference.

Contrast - adjusts the difference between light and dark areas or contrast for the selected camera according to your preference.

Saturation - adjusts the amount of color for the selected camera.

Hue - adjusts the dominant color for the selected camera.

Quality (All) - adjusts the video quality for all cameras. The default setting is good. We recommend that you do not set this to best to avoid using up a significant amount of hard disk space.

Max Connection - limits the maximum number of connections allowed to access the VPON DVR over the Internet. Set to zero to allow the maximum number of connections.

Max Bandwidth - sets the maximum bandwidth in bytes/second that can be used by users connecting to the VPON DVR over the Internet.

Max Conn. Bandwidth - sets the maximum bandwidth allocated for each connection to the VPON DVR. This is the maximum bandwidth divided by the maximum connections.

Audio – Enable or disable audio function

To change the video parameters:

1.	Use the Up and Down arrow buttons on the remote control to highlight the option.

2.	Use the Left and Right arrow buttons to change the value assigned to the setting.

3.	Press the OK button to confirm the new settings.

Each camera has its own video screen. To move between the screens for each camera, use the Prev Page and Next Page buttons on the remote control.

PTZ Control

Select the type of PTZ cameras used and set each camera that has one connected to Yes. The timeout value represents the time given to the cameras to respond to a command. Don’t forget to setup PTZ ID.

You can set and recall up to 6 preset configurations for each camera using the following buttons in the PTZ control section:

Preset - save the current pan, tilt, zoom, and focus settings in one of 6 preset configurations.

Recall - recall the selected preset configuration.

ZOOM - adjust the zoom to provide a more closer or wider view of the subject.

FOCUS - adjust the focus of the camera.

Speed - Adjust the speed of the selected PTZ camera.

Pan and tilt - Adjust the pan and tilt of the selected camera.

4.3.3  Disks

Use the Disks menu to add and remove network disks from the system. Network disks can be used to provide extra storage to extend recording time.

To add a network disk:

1.	Highlight the disk you want to add and press the OK button.

2.	When the Add Disk screen is displayed, press the OK button.

The Add Network Disk menu is displayed, enabling you to set the IP address and volume size of the disk, test its validity, and save the disk parameters.

To enter an IP address:

1.	Use the Up and Down arrow buttons on the remote control to highlight the IP/Addr option and press the Right arrow button to confirm the choice.

2.	Enter an IP address using the number buttons on the remote control.

Note: You cannot enter a period (dot) between the fields in the IP address using the remote control keypad. Use the on-screen keypad to enter the dots.

3.	Press the OK button to confirm the new address.

To enter a volume name for the new disk:

1.	Use the Up and Down arrow buttons on the remote control to highlight the Volume option and press the Right arrow button to confirm the choice.

2.	Use the on-screen keyboard to enter a volume label for the new disk.

To test a network disk:

1.	Use the Up and Down arrow buttons on the remote control to highlight the Test! option.

2.	Press the OK button to confirm the choice.

The test result will appear on the screen.

To save disk parameters:

1.	Use the Up and Down arrow buttons on the remote control to highlight the Save! option.

2.	Press the OK button to confirm the choice.

4.3.4	TV output

The VPON DVR can be connected to a VGA monitor or a TV to view the camera video signal. Use the TV Output menu to set the following parameters for the video output port:

Device - sets the output to VGA or TV.

Standard - sets the signal format for the output device. Select from NTSC, PAL, SECAM, and so on.

Output to - sets the video output to the composite or S-Video signal.

Delay - sets the delay between consecutive camera signals when displaying all the camera signals automatically.

Camera (From) - specifies the first camera signal displayed in the automatic cyclic camera mode.

Camera (To) - specifies the last camera signal displayed in the automatic cyclic camera mode.

Screen Split - sets the number (1, 4, 9, or 16) of cameras to be shown on the local screen.

Time Stamp - Enables and specifies the position of the time stamp on the screen.

Time Format - Specifies the format of the time stamp.

Use the Up and Down arrow buttons on the remote control to select any one of the displayed parameters and use the Left and Right buttons to cycle through the available options. Press the OK button when parameters are set correctly.

4.3.5	OSD text

A line of text can be displayed with the video from each camera connected to the VPON DVR. Use the OSD Text menu to enable the text and alter it. You can also set the position of the text using the X and Y parameter values. Increasing the value of Y moves the text down the screen. Increasing the value of X moves the text to the right.

To alter the text position:

1.	Use the Up and Down arrow buttons on the remote control to highlight X or Y.

2.	Use the Right button to increase the value and the Left button to decrease the value.

3.	Press the OK button when the value is correct.

Use the on-screen keyboard to alter the OSD text as follows:

1.	Use the Up and Down arrow buttons on the remote control to highlight the Text option.

2.	Use the Right arrow button to confirm.

3.	Press the keypad button on the remote control to display the on-screen keyboard.

4.	Use the Arrow buttons to highlight the letters you need for your text and press the OK button to choose them.

5.	When your text is complete, press the Keypad button to remove the on-screen keyboard.

6.	Press the OK button to save the new text.

4.3.6	TCP/IP

Use the TCP/IP menu to set the IP address of the VPON DVR, the gateway, the subnet mask, the DNS server IP address, and the HTTP port. If you want to use the VPON DVR on your network, see your network administrator to obtain a valid IP address for the VPON DVR.

To change the TCP/IP parameters:

- Select the field to change using the Up and Down arrow buttons.

2.	Press the Right arrow button to confirm.

3.	Use the number pad on the remote control to enter a new value

4.	Press the OK button to confirm the new value.

5.	Select the Save&Reboot option using the Up and Down arrow buttons and press the OK button to restart the system and bring the changes into effect.

#### Multicast

After starting the function of “Multicast”, system will transfer video to the network automatically

## Firewall

The function of Firewall is to protect your DVR system from Internet attacks so that you can hook up your VPON to Internet safely

### DROP / ACCEPT

Default to selecte as DROP or ACCEPT. If default is DROP, that means rules in the Accept List are accepted and all others are rejected. On the contrary, If default is ACCEPT, that means rules in the Drop List are rejected and all others are accepted

### Add / Edit

Adding or edit an IP address. “mask” means domain address. For example, when key in IP:10.11.100.151，Mask:255.255.255.0，this means IP address from 10.11.100.1 to 10.11.100.255 will be blocked，totally 255 IP addresses。

HTTP: When default is DROP and HTTP is NO (not Apply), this means the HTTP port of this IP is rejected. On the contrary，if default is ACCEPT and HTTP is NO (not Apply)，that means the HTTP port of this IP address is opened。

FTP: same as HTTP。

### DROP / ACCEPT LIST

Rules in the list are checked by top down sequence. When any one of the rule is satisfied, checking will NOT go down.

For Example:

Default Rule ACCEPT

1. 10.11.100.151/24 HF  (means IP coming from 10.11.100.* are all rejected.)

2.10.11.100.154/32     (means ports from 10.11.100.154 are all rejected EXCEPT HTTP and FTP )

If you use 10.11.100.154 to access VPON DVR web page, it will fail.

When the sequence of Rule1, Rule2 are exchanged and Default Rule is ACCEPT

1.10.11.100.154/32

2. 10.11.100.151/24 HF

Then, if you use 10.11.100.154 to access VPON DVR web page，it will work OK.

### Move Up- Move a specific Rule one row up

### Move Down- Move a specific rule one row down.

### Delete - Delete a specific rule in Rule table.

### Delete All- Delete all rules in Rule table.

4.3.7	Account

Use the Account menu to set up an administrator name and password as well as user name and passwords. The default administrator name is Webmonitor and the default password is OYO. You can also use this menu to set up to 16 additional users, their passwords and their permissions on the system.

Note: It is important that you set up a user name and password. The user is different from the administrator and has access to only the video. Use the More Users function to alter the authorities of users who access the VPON DVR from the Internet.

To set an Admin/User name or password:

1.	Use the Up and Down arrow buttons to select the field to change.

2.	Press the Right arrow button to confirm.

3.	Press the Key Pad button on the remote control to display the on-screen keyboard.

4.	Use the Arrow buttons to select the letters you require and the OK button to confirm.

5.	When the field is correct, press the Key Pad button to turn off the on-screen keyboard.

6.	Press the OK button to confirm.

To set up additional users and their permission rights:

- Use the Up and Down arrow buttons to select the More Users choice.

2.	Press the Right arrow button to confirm.

3.	Use the Arrow buttons to select the Name and Pass options and modify these fields in the same manner as in the previous procedure.

4.	Use the Up and Down buttons to select any cameras from Camera1 through Camera16.

5.	Press the Right arrow to select Yes to allow the user to access and control the selected camera.

6.	Use the Up and Down buttons to select the PTZ option.

7.	Press the Right arrow button to select Yes to enable PTZ control of the selected cameras for the user.

8.	Use the Up and Down buttons to select the Playback option.

9.	Press the Right arrow button to select Yes to enable the user to record and play back the signal from the selected cameras.

10.Use the Up and Down buttons to select the Audio option.

Note: Audio can be disabled for live viewing and playback of recordings using the Audio Disable function.

11.Press the Right arrow button to select Yes to enable the user to hear the live audio from the selected cameras.

4.3.8	Audio

Use the Audio menu to set up the audio parameters for the VPON DVR. The configurable parameters areas follow:

- Speaker - adjusts the speaker output level (volume).

Note: Make sure that audio is enabled when recordingsignals from the cameras attached to the VPON DVR.

4.3.9	Serial ports

The VPON DVR can be connected to other serial devices such as external modems, PTZ cameras, camera control devices, text input devices, or USB serial ports, using two serial ports - COM1 and COM2. The RS232 standard is used for these ports. Use the Serial Ports menu to set the following parameters:

COM1 Device - selects the type the serial device.

COM1 Speed - sets the speed (1200 to 115200 bps) for the serial device.

COM1 Flow Control - enables or disables the flow control for the device.

COM1 Data Bits - Sets the number of data bits (6, 7, or 8) for the device.

COM1 Parity - Sets odd or even parity.

COM1 Stop Bits - Sets 1 or 2 stop bits for the device.

To change the serial port parameters:

1.	Use the Up and Down buttons to select the field to change.

2.	Use the Left and Right arrow buttons to cycle through the available options for that field.

3.	Press the OK button when the field is set correctly.

Note: You can use the Prev Page and Next Page buttons on the remote control to toggle between the COM1 menu and COM2 menu.

4.3.10	System information

Use the System Information option to display the System Information screen and view information about the boot disk, TCP/IP parameters, the MAC ID of the device, firmware version and date of installation, model number, and hard disk properties.

4.3.11  Date/Time

Use the Date/Time option to set the year, month, day, hour, minute, second, and time zone in which the VPON DVR is installed.

To set the time or time zone:

1.	Use the Up and Down arrow buttons to select the field to change.

2.	Use the Left arrow to decrease the value and the Right arrow to increase the value for that field.

3.	Press the OK button when the values are set correctly.

Note: The VPON DVR reboots to save the changes, if you modify the time zone.

4.3.12	Alarm

Use the Alarm menu to define what constitutes an alarm condition and how the VPON DVR reacts when such a condition occurs. You can configure an alarm condition to be generated when motion is detected by the camera, if the video signal is lost from a camera, if the disk space is too low, or if there is a disk error. Once the alarm is generated, you can set the alarm notification through a beep, a message, a video popup or E-mail.

The Alarm menu has the following submenus:

E-Mail - enables you to set up an E-mail address and verify it is working correctly.

Video Popup (sec) - sets the time for which a camera is displayed on your monitor when an alarm is generated.

Voice Call – this function is not supported.

Set Action - sets the action that will generate an alarm condition for each camera.

To set Motion Detection：

1.	Use the Up and Down arrow buttons to select the Motion Detection option from the Alarm menu and press the OK button.

2.	When the motion detection menu appears, uses the Up and Down Arrow buttons to select one of the following options:

Enable- configures the motion detection feature for each camera.

Sensitivity -sets the sensitivity value for detecting motion.

Set Detect Area -sets the area in the camera’s field of vision over which motion is detected. This option is displayed only after you set the Enable option to Yes.

3.	Select the Enable field and use the Left and Right arrow buttons to toggle between Yes, to enable the motion detection feature, and No, to disable it.

4.	Select the Sensitivity choice and use the Left/Right arrow buttons to decrease/increase the sensitivity. We recommend that you keep the sensitivity level to its default setting. If the sensitivity level is adequate, an orange square is displayed in the upper left-hand corner of the screen when motion is detected.

5. Select the Set Detect Area option and press the Right arrow button to configure the area as follows:

a. The highlighted area in red is the area monitored by motion detection. Use the Arrow buttons to move the cursor.

b. Press the OK button to cut off any areas where you want to disable motion detection. If you want motion detection enabled in the entire field of vision of the camera, leave the screen completely red.

c. Press the Cancel button to return to the previous menu when done.

6. Select the Save option and press the OK button to save all your changes.

7. Use the Prev Page and Next Page buttons to move to the motion detection menus for all connected cameras.

8. Press the OK button when settings are correct.

To enter E-mail details:

1.	Select the E-Mail option from the Alarm menu.

2.	Press the Right button to confirm. The E-Mail submenu is displayed.

3.	In the E-Mail menu, use the Up and Down buttons to select a field from the following:

Addr - Type the name of your mail server. See your network administrator for details about the address of your mail server.

Name - Type your user name.

Pass - Type your password.

Sender - Type the e-mail address of the sender.

Test - Checks the validity of the provided E-mail address by sending E-mail.

4.	Press the Right arrow button to confirm your selection.

5.	Press the Key Pad button on the remote control to display the on-screen keyboard.

6.	Use the Arrow keys to select the letters you need to complete the field and press the OK button to confirm.

7.	When the field is complete, press the Key Pad button to turn off the on-screen keyboard.

8.	Press the OK button to save your changes.

Video Popup

Use the Video Popup menu to set up the time period for which the video from the selected camera is displayed on your screen. The default screen is 10 seconds.

To set the video popup time:

1.	Use the Up and Down arrow buttons to select the Video Popup option from the Alarm menu.

2.	Use the Left arrow button to decrease the popup time and the Right arrow button to increase the time

3.	Press the OK button to save your settings.

Set Action

Use the Set Action option to define what action is taken by the VPON DVR when an event occurs. The following options are available:

Video Lost - generates an alarm when the video from the selected camera is not available.

Disk Space Low - generates an alarm when the disk space is insufficient.

Disk Error - generates an alarm if there is an error while accessing the disk.

To configure events using the Set Action option:

1.	Use the Up and Down arrow buttons to select an event from the list.

2.	Press the Right arrow button to display the submenu for the event with the following options:

Message - displays a message on your screen when the event occurs.

Beep - sounds a beep when the event occurs.

E-Mail - sends an E-mail to the specified address when the event occurs.

Voice Call – this function is not supported.

Video Popup - displays the video from the selected camera on your screen.

3. Use the Up and Down buttons to select the desired action from the following options:

4. Press the Right arrow button for the required action to view the submenu.

5.	After enabling the action, set up the Start and End Hour, Minute, and Second to make the feature active during the specified time interval of the day. The default settings have the feature enabled throughout the whole day.

6.	If you selected the E-Mail option as the desired action, you have to set up the recipient of the E-mail, the subject and the text matter of the E-mail. You can specify which of the 16 cameras status is in the E-mail.

4.3.13	ISP

If you are connecting to the Internet using dial-up, enter your ISP details using the ISP option from the System Setup menu. Set up the following options:

Phone No. - enter the telephone number that is dialed by the dial-up modem.

Name - enter the user name of your dial-up account.

Pass - enter the password for your dial-up account.

TCP/IP - set up automatic or manual TCP/IP. If the VPON DVR is connecting through a DHCP server, an IP address is assigned to it automatically. If you want to set the IP address of the device manually, select the Manual option.

To enter ISP details:

1.	Use the Up and Down arrow buttons to select a field to change.

2.	Press the Right arrow button to confirm.

3.	Press the Key Pad button on the remote control to display the on-screen keyboard.

4.	Use the Arrow buttons to select letters and numbers from the on-screen keyboard and use the OK button to confirm.

5.	When the field is correct, press the Key Pad button to turn off the on-screen keyboard

6.	Press the OK button to confirm the new field.

4.3.14	Registry server

The Registry Server menu enables you to set up the registry server for your device. If you are using a dynamic IP address for the VPON DVR, you can set up the device to post its IP address to the VPON registry server. You can then look up your IP from the MAC address or server name.

To enter Registry Server details:

1.	Use the Up and Down arrow buttons to select a field to change.

2.	Press the Right arrow button to confirm.

3.	Press the Key Pad button on the remote control to display the on-screen keyboard.

4.	Use the Arrow buttons to select letters and numbers from the on-screen keyboard and use the OK button to confirm.

5.	When the field is correct, press the Key Pad button to turn off the on-screen keyboard.

- Press the OK button to confirm the new field.

4.3.15	Running mode

Use the Running Mode menu to switch between Network Mode and ISP Mode, and between Mouse Mode and IR Remote Control Mode.

4.3.16	Algorithm

You can change the compression algorithm used by the VPON DVR to record video.

To set up the algorithm:

1.	Use the Up and Down arrow buttons to select a field to change from the Algorithm menu.

2.	Use the Right and Left arrow buttons to toggle the available options.

3.	Press the OK button when the fields are set correctly.

4.3.17	NTP setup

Use the NTP (Network Time Protocol) Setup function to synchronize the real-time clock on the VPON DVR with the atomic clock. Before using this feature, make sure that the correct date, time and time zone have been set up. Also make sure that the DNS IP number has been set up under the TCP/IP settings. The following options are available in the submenu:

Enable - enables the NTP function to synchronize the real-time clock.

Period (Hour) - sets up the time interval after which the time is re-synchronized.

Server - selects the NTP server from several pre-defined choices.

Edit NTP Server - enables you to choose your own NTP server.

Test&Save? - tests the server and saves the changes.

To change NTP settings:

1.	Use the Up and Down arrow buttons to select a field to change from the NTP menu.

2.	Press the Right button to confirm.

3.	Press the Key Pad button on the remote control to display the on-screen keyboard.

4.	Use the Arrow buttons to select letters from the on-screen keyboard and use the OK button to confirm.

5.	When the field is set correctly, press the Key Pad button on the remote control to turn off the on-screen keyboard.

6.	Press the OK button to save changes.

4.3.18	Default settings

Use the Default Settings option to reset all configurable parameters to their factory default settings. When the Default Settings option is chosen from the System Setup menu, a message is displayed asking you to confirm. Press the OK button to confirm, or the Cancel button to cancel the operation.

4.3.19	Revise firmware

Use the Revise Firmware option to upgrade the firmware in the VPON DVR. The firmware can be revised through an external read-write CD drive, a USB flash drive, or a USB thumb drive. Connect the device to the VPON DVR and then use this option.

4.4	Record setup

Use the Record Setup menu to set all the record options for the VPON DVR. There are five Record Setup submenus - DVR, Record Schedule, Motion Detection, GPI trigger, and Pre-Alarm. This section covers each submenu in turn.

4.4.1	DVR Setup

Use the DVR menu to set the DVR (Digital Video Recorder) mode. Choose Cyclic Recording to keep recording on the hard disk continuously. Choose Auto Stop to stop recording when the disk is full.

You can set an alarm to be generated when the hard disk is full and send notification through a beep, E-mail, or text message.

To change the DVR mode:

1.	Select the Mode option from the DVR submenu using the Up and Down arrow buttons on the remote control.

2.	Use the Left and Right buttons to select either Cyclic Recording, or Auto Stop.

3.	Press the OK button to save changes.

4.4.2	Record schedule

You can set up to 16 record schedules that automatically record at given times and days.

To configure a record schedule:

1.	From the Record Schedule menu, pick a schedule to change from the displayed list, using the Up and Down arrow buttons.

2.	Press the Right button to display the parameters for that schedule.

3.	Use the Up and Down arrow buttons to select a field to change, from the following options:

Enable -enables the recording schedule.

Mon through Sun - enables recording on the specified days of week.

Start Hour/Minute/Second - sets the exact time when the recording starts.

Stop Hour/Minute/Second - sets the exact time when the recording ends.

Camera 1 through 8 - sets the recording mode for the selected camera. Select Normal Record to record continuously. Select Motion Detect to record only when motion is detected. Select OFF to disable recording.

Audio - enables recording of sound along with the video signal.

Frame Rate Setup - sets the frame rate for recording. Set Auto Speed to Yes and the device evenly divides the recording frame rate amongst all cameras. If set to No, you can manually set up the individual frame rates for all cameras.

4.	Use the Right and Left arrow buttons to change the value of that parameter.

5.	Use the Prev Page and Next Page buttons to move to the previous or next page.

6.	Press the OK button when all parameters are set.

Weighted Rec - sets special recording frame rate when an alarm of one of camera was triggered.

4.4.3 Motion Detection

1.	Use the Up and Down arrow buttons to select the Motion Detection option from the Alarm menu and press the OK button.

2.	When the motion detection menu appears, uses the Up and Down Arrow buttons to select one of the following options:

Enable- configures the motion detection feature for each camera.

Sensitivity -sets the sensitivity value for detecting motion.

Set Detect Area -sets the area in the camera’s field of vision over which motion is detected. This option is displayed only after you set the Enable option to Yes.

3.	Select the Enable field and use the Left and Right arrow buttons to toggle between Yes, to enable the motion detection feature, and No, to disable it.

4.	Select the Sensitivity choice and use the Left/Right arrow buttons to decrease/increase the sensitivity. We recommend that you keep the sensitivity level to its default setting. If the sensitivity level is adequate, an orange square is displayed in the upper left-hand corner of the screen when motion is detected.

5. Select the Set Detect Area option and press the Right arrow button to configure the area as follows:

a. The highlighted area in red is the area monitored by motion detection. Use the Arrow buttons to move the cursor.

b. Press the OK button to cut off any areas where you want to disable motion detection. If you want motion detection enabled in the entire field of vision of the camera, leave the screen completely red.

c. Press the Cancel button to return to the previous menu when done.

6. Select the Save option and press the OK button to save all your changes.

7. Use the Prev Page and Next Page buttons to move to the motion detection menus for all connected cameras.

- Press the OK button when settings are correct.

4.4.4  GPI Trigger

Use the GPI Trigger function to enable recording video from specified cameras when triggered by an internal alarm (GPI) connected to the system.

4.4.5  Pre- Alarm

Use the Pre-Alarm function to determine how many frames are recorded before an alarm event occurs. Each camera can be set separately and you can record up to 300 frames.

To set the Pre-Alarm frame capture:

- Use the Up and Down arrow buttons to select Pre-Alarm option from the Record Setup menu.

2. Press the OK button to display the Pre-Alarm menu.

3. Use the Up and Down arrow buttons to select the camera to change from the list.

4. Press the Right arrow button to confirm.

5. Use the number buttons on the remote control to enter a new value.

6. Press the OK button when the value is set correctly.

4.5	Playback

Use the Playback function to display a list of all recordings stored on the VPON DVR. Use the Up and Down arrow buttons to select a recording to play back and press the OK button to confirm. The chosen track is played. Use the Prev Page and Next Page buttons on the remote control to move between pages.

# 4.5.1  Playback list

Use the Playback list function to display a list of all recordings stored on the VPON DVR as follows.

1.	Use the Up and Down arrow buttons to select a recording to play back. The alarm condition which caused the file to be recorded is indicated in the list. For instance, in the screen shot shown here, the letter M next to a file indicates that the file was recorded after motion was detected by the camera. indicates that the file was recorded after a GPIO event. An asterisk (*) before the date indicates that the file is locked.

2.	Press the OK button to confirm. The chosen recording is played back.

3.	Press the Display button on the remote control to display various properties of the recorded file. The date “A one hour time line is displayed at the bottom of the screen with the moving orange line indicating the status of playback.

Recorded Data - The aqua colored line represents how much of the one hour time interval was recorded. A full blue line means that the whole hour interval was recorded.

Motion Detection - The screen shows where Motion Detection triggers were generated. The red bar shows the triggers. To move to the next motion detection, press the Right/Left arrow buttons.

GPI - The screen shows where alarm triggers were generated. The red bar shows the triggers. To move to the next alarm trigger event, press the Right/Left arrow buttons.

# 4.5.2  Searching for a file

The VPON DVR playback list can be searched for a specific file. You can conduct the search based on time, event, or text.

To search for a recorded file:

- While viewing the play list, press the Menu button to display the menu.

2.	Select one of the following options using the Up or Down arrow buttons:

Search by Time - searches the list of recorded files for the specified recording time.

Search by Event - searches the list of recorded files for the specified event such as a GPIO trigger or Motion Detection that occurred within the specified time interval.

3.	Press the Right arrow button to view the submenu for the selected option.

4.	Select each of the options such as Year, Month, Day, and so on using the Up or Down arrows.

5.	Modify the settings as required using the Left and Right arrow buttons.

Note: If you select the Search by Event option, choose an event from GPI 1~8.

6.	Press OK when done to conduct the search.

4.5.3  Locking files

To prevent accidental deletion of recorded files, you can lock a file. This also prevents over-writing of the file when the recording mode is set to Cyclic.

To lock a file:

1.	While viewing the play list, use the Up/Down arrow buttons to select the desired file.

- Press the Menu button to display the menu.

3.	Select the Lock File! option using the Up or Down arrow buttons:

4.	Press the OK button to confirm.

The file is locked and cannot be deleted or over-written. The locked file is represented by an asterisk (*) next to it. To unlock a file, follow the same procedure and select the Unlock File! option from the menu.

4.5.4  Backing up the data

If the VPON DVR is equipped with CD/RW drive, you can back up your recorded files to a CD or USB hard drive.

To back up your data:

1.	While viewing the play list , press the Menu button to display the menu.

2.	Select the Backup option using the Up or Down arrow buttons.

3.	Press the OK button to confirm.

The VPON DVR detects if there is a backup device connected to it and starts backing up data.

Note: The VPON DVR will begin to collect data as soon as the above screens are displayed. When the required data has been collected, the backup can begin.

4.6	System Shutdown

Use the System Shutdown option to shut down or reboot the system. To shut down or reboot the system:

1.	Use the Up and Down arrow buttons to select either Shutdown or Reboot.

2.	Press the OK button to confirm.

3.	Press the OK button to confirm the operation again, or press the Cancel button to cancel the operation.